CO2 APPLICATION-BASED ASSIGNMENT

SMART MANUFACTURING: AI-DRIVEN PREDICTIVE MAINTENANCE SYSTEM

ECA58 – IoE AND AI DRIVEN SYSTEM 
UNIT II – DATA AND INTELLIGENCE IN IoE

Submitted by

Jeeva R

Register Number: 1912260266

Course Outcome: CO2 – Analyse IoE data lifecycle processes, analytics, big data and AI/ML techniques for intelligent real-time IoE solutions.

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 1

Executive Summary

This report proposes a hybrid Industrial Internet of Everything (IoE) predictive-maintenance system for a medium-scale automotive component plant containing 200 CNC machines, presses and robotic assembly stations. Sensors and PLCs feed industrial edge gateways. The gateways validate and preprocess signals, extract compact features, run local AI inference and generate alarms. A plant platform stores features, events and selected raw diagnostic records. Spark supports historical preparation and streaming analytics; dashboards support operators and maintenance managers. The design prioritizes local response because critical alerts must be produced within 100 ms and basic detection must continue for 30 minutes without cloud connectivity.

The engineering decision is to transmit features and events by default, not continuous raw vibration. Under the stated high-resolution assumptions, raw vibration is 6.4 MB/s (51.2 Mb/s) or 368.64 GB per 16-hour production day. A compact feature packet every 10 seconds reduces the illustrative upstream volume by approximately 99.86%, exceeding the 90% target. Final approval is recommended subject to a staged pilot that verifies model performance, latency, storage and safety integration.

1. Stage 1 – Industrial IoE Data Lifecycle Evaluation

1.1 Existing arrangement and lifecycle gaps

| Lifecycle stage | Present situation / gap | Proposed improvement |
| --- | --- | --- |
| Acquisition | PLCs collect machine status, but vibration and condition channels are not integrated into a unified health dataset. | Add vibration, temperature, current, pressure and acoustic sensors; synchronize timestamps and machine IDs. |
| Transmission | Ethernet/Wi-Fi coverage is uneven; interference and congestion can interrupt data flow. | Use wired industrial Ethernet where practical, industrial Wi-Fi in suitable areas and segmented gateway connections. |
| Storage | Local dashboards do not provide a defined 30-day raw / 2-year feature retention policy. | Use gateway ring buffers, plant time-series/object storage and lifecycle-managed archive storage. |
| Preprocessing | No consistent filtering, data-quality checks, windowing or feature extraction. | Perform validation, filtering, windowing and feature extraction at the edge. |
| Analytics | Monitoring is descriptive; there is no validated predictive model or health score. | Deploy anomaly detection/fault classification at the edge and batch learning centrally. |
| Decision/action | Alarms are not linked systematically to maintenance work orders or outcomes. | Use NORMAL/WARNING/CRITICAL rules, work-order integration and maintenance feedback. |
| Governance | Machine identity, model versions, access control and drift monitoring are not specified. | Use device identity, role-based access, signed model packages, audit logs and drift monitoring. |

1.2 Raw data generation calculation

Given: 200 machines; 8 channels per machine; 2,000 samples/second/channel; 16 bits/sample = 2 bytes/sample. Assume all channels run continuously at the stated high-resolution rate during 16 production hours per day, with no protocol overhead or compression.

Per machine: 8 × 2,000 × 2 = 32,000 bytes/s = 32 kB/s.

Plant: 200 × 32,000 = 6,400,000 bytes/s = 6.4 MB/s = 51.2 Mb/s.

Per production day: 6,400,000 × 16 × 3,600 = 368,640,000,000 bytes = 368.64 GB/day (decimal units).

30 days, if all raw data is retained: 368.64 × 30 = 11,059.2 GB ≈ 11.06 TB, before replication, indexes and storage overhead.

1.3 Raw transmission versus edge features

| Indicator | Raw continuous approach | Feature/event approach target |
| --- | --- | --- |
| Upstream data | 6.4 MB/s baseline | At least 90% reduction; proposed calculation gives about 99.86%. |
| Alert latency | Depends on network/cloud round trip; may exceed 100 ms. | Local inference and alarm target below 100 ms. |
| Network resilience | Cloud/network loss interrupts centralized analysis. | At least 30 minutes of local detection and buffering. |

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 2

| Indicator | Raw continuous approach | Feature/event approach target |
| --- | --- | --- |
| Storage | Up to 11.06 TB for 30 days at full-rate assumptions. | Keep 30-day raw data selectively; retain feature/event data for 2 years. |
| Maintenance outcome | Reactive maintenance continues until failure. | Target fewer unplanned stops; verify against pilot baseline. |
| False alarms | No model-based baseline available. | Track false alarms per machine-week and tune threshold on validation data. |

Engineering judgement: Continuous raw upload is not recommended. It consumes bandwidth and central storage, makes the 100 ms requirement dependent on network conditions and weakens operation during outages. Keep short rolling raw buffers locally and upload raw segments only around detected events, for selected machines or during planned diagnostics. Transmit features, health scores, alarms and maintenance events continuously.

2. Stage 2 – Data Acquisition and Edge Preprocessing

2.1 Sensor selection

| Maintenance task | Primary measurements | Reason |
| --- | --- | --- |
| Bearing fault | High-rate vibration; temperature; acoustic level | Vibration reveals impact, imbalance and frequency components; temperature and acoustic changes provide corroboration. |
| Motor degradation | Motor current; vibration; temperature; operating state | Current trends indicate load/electrical changes; vibration and temperature reveal mechanical/thermal stress. |
| Hydraulic abnormality | Pressure; temperature; operating state; acoustic level | Pressure variation and temperature indicate leakage, blockage, overheating or unstable hydraulic behaviour. |

2.2 Edge preprocessing sequence

1. Timestamp and validate: attach machine/channel identity, check sensor range, sequence number and clock synchronization. 2. Missing/duplicate handling: flag gaps, remove duplicates and mark low-confidence windows rather than silently inventing values. 3. Filtering: apply an appropriate anti-alias/noise filter before feature calculation. 4.

Windowing: divide vibration into one-second windows with a defined overlap. 5. Normalization: scale features using training-set statistics and retain the same parameters at inference. 6. Feature extraction: calculate time- and frequency-domain features. 7. Event-triggered raw retention: preserve a raw pre/post-event segment for diagnosis. 8. Packetization: compress and transmit features and events.

2.3 One-second vibration feature set

| Feature | Meaning / maintenance value |
| --- | --- |
| RMS | Overall vibration energy; rises with imbalance, looseness or bearing damage. |
| Peak amplitude | Largest sample magnitude; captures impulsive events. |
| Peak-to-peak | Difference between maximum and minimum; indicates large excursions. |
| Crest factor | Peak divided by RMS; sensitive to repetitive impact faults. |
| Variance / standard deviation | Measures signal spread and changing vibration level. |
| Kurtosis | Highlights impulsive, non-Gaussian vibration often associated with bearing defects. |
| Dominant frequency | Largest spectral component; helps identify rotational or fault-related frequency changes. |
| Band energy | Energy in selected frequency bands; supports bearing and gear fault diagnosis. |

2.4 Feature packet and reduction calculation

Assume 8 numeric features per channel, each stored as a 4-byte float. Add 16 bytes of timestamp, 8 bytes ofmachine/channel identifiers and 8 bytes for status/quality/version metadata. Packet size = (8 × 4) + 16 + 8 + 8 = 64 bytes per channel per 10-second packet.

There are 200 × 8 = 1,600 channels. Packets per second = 1,600 / 10 = 160. Feature traffic = 160 × 64 = 10,240 bytes/s = 10.24 kB/s. Daily feature traffic = 10,240 × 57,600 = 589,824,000 bytes ≈ 0.590 GB/day.

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 3

Reduction = [1 − (10,240 / 6,400,000)] × 100 = 99.84%. This exceeds the required 90%. The estimate excludes transport headers, retransmissions and additional process-variable packets; even with overhead, there is substantial margin. The packet should contain machine ID, channel ID, timestamp, sequence number, feature values, machine state, quality flags, model/feature schema version and alarm/event flag.

2.5 Acquisition-to-edge block diagram

Sensors / PLCs
↓
Industrial Ethernet / Wi-Fi / fieldbus
↓
Edge gateway (timestamp, identity, validation)
↓
Filtering → missing/duplicate checks → 1-second windowing
↓
Feature extraction (RMS, kurtosis, crest factor, FFT bands, etc.)
↓
Local feature store + short raw ring buffer
↓
Local AI inference → NORMAL / WARNING / CRITICAL alarm
↓
Transmit compact features, events and selected raw snippets to plant platform

3. Stage 3 – Analytics and Visualization

3.1 Dashboard tool decision matrix

Weights reflect the need for live industrial time-series monitoring: real-time capability 25%, time-series suitability 20%, alert integration 15%, scalability 15%, customization 10%, cost 10% and deployment ease 5%. Scores are 1 (weak) to 5 (excellent); they are planning judgements to be confirmed during procurement.

| Tool | Real-time 25 | Time-series 20 | Alerts 15 | Scale 15 | Custom 10 | Cost 10 | Ease 5 | Weighted / 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Grafana | 5 | 5 | 5 | 4 | 5 | 5 | 4 | 4.75 |
| Power BI | 3 | 3 | 4 | 4 | 3 | 3 | 4 | 3.35 |
| Tableau | 3 | 3 | 3 | 4 | 5 | 2 | 3 | 3.25 |

Grafana is selected for the operator's near-real-time machine-health view because it is well suited to time-series dashboards and alerting. Power BI can be used for management summaries and business reporting. Tableau is a capable analytical visualization option, but its cost and deployment fit should be assessed against the existing enterprise environment.

3.2 Dashboard designs

| Operator real-time view | Maintenance-manager trend view |
| --- | --- |
| Plant overview: 200 machine tiles with NORMAL/WARNING/CRITICAL status; active alarms; last update age; gateway connectivity. | Plant availability and downtime trend; failures by machine family; monthly maintenance cost; planned vs emergency work. |
| Per-machine drill-down: vibration RMS trend, spectral band energy, temperature/current/pressure gauges, operating state and health score. | Machine health score distribution; anomaly frequency heatmap; remaining-useful-life trend with confidence; maintenance history timeline. |
| Alarm details: machine ID, fault class, severity, timestamp, confidence, recommended action and acknowledgement. | Drill-down by machine, line, component, fault type, work order, failure date and post-maintenance outcome. |

3.3 Visualization selection and data quality

| Information | Recommended visualization | Reason |
| --- | --- | --- |
| Vibration trend | Time-series line chart with warning threshold | Shows trend, transient spikes and threshold crossings over time. |

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 4

| Information | Recommended visualization | Reason |
| --- | --- | --- |
| Machine-health score | Gauge or bullet chart plus trend line | Communicates current condition and whether it is approaching a threshold. |
| Anomaly frequency | Bar chart / calendar heatmap | Compares machines and reveals recurring time patterns. |
| Remaining useful life (RUL) | Trend line with uncertainty band | Shows predicted life and uncertainty, not just a single number. |
| Maintenance history | Event timeline / table | Preserves chronological work-order and component-replacement context. |

Data-quality monitoring checks missing timestamps, duplicate sequence numbers, delayed arrival, out-of-range values, flat-lined sensors, sudden impossible jumps and clock drift. Each record receives a quality flag. Low-quality windows are excluded or down-weighted; operators are alerted to sensor faults. Poor data can create false warnings, hide real failures or bias model training.

Sensor event → data-quality gate → abnormal trend / AI score → severity rule → operator acknowledgement →maintenance work order → inspection / repair → record actual finding and replaced part → compare prediction with outcome → model monitoring and retraining.

4. Stage 4 – Big-Data Handling with Hadoop and Spark

4.1 Hadoop versus Spark evaluation

| Criterion | Hadoop ecosystem | Apache Spark | Plant decision |
| --- | --- | --- | --- |
| Batch processing | Strong distributed storage and batch ecosystem. | Fast distributed batch processing with in-memory options. | Use both where appropriate; Spark for feature preparation and model-training datasets. |
| Iterative analytics | Disk-oriented MapReduce can be slower for repeated iterations. | Efficient for repeated transformations and ML preparation. | Prefer Spark for iterative analytics. |
| Streaming | Typically requires complementary ingestion/streaming components. | Structured Streaming supports near-real-time processing. | Use Spark streaming for plant-level monitoring, not the sub-100 ms safety-critical edge alarm. |
| Fault tolerance | Distributed storage replication and job recovery. | Lineage-based recovery and checkpointing. | Use replicated storage, checkpoints and backups. |
| Resources / latency | Can require substantial cluster resources; batch latency is higher. | Memory and CPU demand can be significant; latency depends on deployment. | Right-size the cluster and keep immediate inference at gateways. |

4.2 Hybrid storage architecture

Gateway: short rolling raw buffer, local feature/event queue, model and configuration cache; retain enough data for at least 30 minutes of disconnected operation. Plant platform: time-series feature store, event/alarm store, machine metadata, maintenance records and selected raw event snippets. Long-term storage: 30-day raw diagnostic retention where selected and at least two years of summarized features/events, model versions and audit records. Apply retention policies and encryption.

4.3 Storage sizing

If all 200 machines' high-resolution raw streams were retained for 30 days, the estimate is 11.06 TB (from Stage 1). Add a planning allowance of 30% for metadata, indexes, replication and operational headroom: 11.06 × 1.30 ≈ 14.38 TB. This is a conservative upper-bound scenario, not the recommended default.

For feature data at 0.590 GB/day, two years (730 days) requires 0.590 × 730 ≈ 430.7 GB before replication and metadata. With a 30% allowance, plan approximately 560 GB. Actual capacity depends on packet overhead, number of features, additional sensors, compression, retention policy and whether all machines operate at the maximum rate.

The estimate may be invalid if: (1) diagnostic sampling is triggered more often or runs beyond 16 hours/day; (2) the number of machines/channels or feature fields grows; (3) replication, backup and retention copies differ from the assumed 30%; or (4) network/protocol overhead and raw event snippets are larger than expected.

4.4 Hadoop/Spark data flow

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 5

PLCs / sensors → gateway validation and feature extraction → message broker / secure ingestion → plant feature & event store 
I selected raw diagnostic objects → 30-day raw retention 
Feature/event history → Hadoop-compatible distributed storage / data lake → Spark batch + streaming preparation →labelled training dataset → model training and validation → model registry → signed deployment to edge gateways

Spark should be used for historical model-training preparation and plant-level stream analytics. It should not be placed in the critical 100 ms alarm path because network, scheduling and cluster delays are less predictable than local inference.

5. Stage 5 – AI Decision System

5.1 ML, deep learning and reinforcement learning

| Approach | Strengths | Limitations for this plant | Decision |
| --- | --- | --- | --- |
| Supervised ML | Works well with engineered vibration/process features; interpretable options; modest compute. | Requires reliable labelled failure/maintenance examples; rare failures create imbalance. | Primary approach: gradient-boosted trees or Random Forest for fault classification, with anomaly detection for unseen behaviour. |
| Deep learning | Can learn complex patterns from raw signals and spectrograms. | Needs more labelled data, compute and careful validation; harder to explain and deploy on constrained gateways. | Evaluate later if pilot data justifies the added complexity. |
| Reinforcement learning | Can optimize sequential actions when a safe reward model exists. | Unsafe to learn maintenance actions by trial and error on live machinery; reward design is difficult. | Not selected as the first approach. |

5.2 Selected AI pipeline

Task: machine fault classification into normal, bearing-related, motor-related and hydraulic-related conditions. Inputs: 1-second vibration features plus temperature, current, pressure, acoustic level and machine operating state. Target: fault class confirmed by inspection/work order. Training data: historical features joined to verified maintenance outcomes; exclude uncertain labels. Validation: split by machine and time, not random windows alone, to reduce leakage; retain a final unseen test period. Use class weighting or careful resampling only within training data.

Candidate algorithms: Random Forest, gradient-boosted trees and a one-class anomaly detector. Compare F1, recall, precision, inference time, memory, explainability and maintenance impact. A boosted-tree model is the proposed primary classifier; a lightweight anomaly detector provides a secondary warning for unfamiliar patterns.

5.3 Metrics for rare failures

Accuracy alone is unsafe when failures are rare: a model can predict NORMAL for nearly every record and still show high accuracy while missing failures. Report per-class precision, recall, F1-score, macro-F1, confusion matrix, false alarms per machine-week, missed critical failures and detection lead time. The acceptance target is macro-F1 ≥ 0.90 on a time-separated validation set, with critical-fault recall separately ≥ 0.95 as a proposed safety-oriented pilot gate. If the supplied 90% requirement is interpreted as either accuracy or F1, macro-F1 is the more defensible primary metric under class imbalance. Thresholds must be calibrated with maintenance engineers.

5.4 Decision logic

| Class | Illustrative rule | Action |
| --- | --- | --- |
| NORMAL | Healthy score ≥ 80; no persistent anomaly; data quality acceptable. | Continue operation; log features and monitor trend. |
| WARNING | Health score 50–79 or repeated anomaly / rising trend; no immediate critical indicator. | Notify operator; schedule inspection; increase sampling and preserve raw diagnostic window. |
| CRITICAL | Health score < 50, high-confidence critical fault, or validated hard-limit condition. | Generate local alarm within 100 ms; follow approved stop/isolation procedure; create urgent work order. |

The numerical health-score boundaries are initial design thresholds and must be tuned on plant data. Operating state and maintenance history modify interpretation: startup and tool-change transients should not be treated as steady-state faults; a recently replaced bearing should have a new baseline.

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 6

5.5 Edge versus cloud inference

| Factor | Central/cloud inference | Edge inference |
| --- | --- | --- |
| Latency | Network-dependent; difficult to guarantee below 100 ms. | Local processing supports deterministic low-latency alarms. |
| Connectivity | Requires reliable upstream connection. | Can continue during network loss. |
| Privacy / data volume | May require sending more data centrally. | Features and selected snippets leave the gateway. |
| Compute | Scales centrally. | Constrained by 4 GB RAM / 20 GB storage per gateway. |
| Updates | Central model update is simpler. | Requires signed, staged and versioned distribution. |
| Role | Training, fleet analytics, dashboards and model management. | Primary real-time inference and local alerting. |

5.6 Weighted model-selection matrix

| Criterion (weight) | Random Forest | Gradient-boosted trees | 1-class anomaly model |
| --- | --- | --- | --- |
| Predictive performance (30%) | 4 | 5 | 3 |
| Inference time (15%) | 5 | 4 | 5 |
| Explainability (15%) | 4 | 4 | 3 |
| Memory footprint (15%) | 4 | 4 | 5 |
| Training complexity (10%) | 4 | 3 | 4 |
| Maintenance impact (15%) | 4 | 5 | 3 |
| Weighted score / 5 | 4.15 | 4.35 | 3.75 |

Scores are design estimates, not measured results. Gradient-boosted trees lead for the proposed labelled classification task; Random Forest is a strong interpretable benchmark. The anomaly detector is complementary rather than a replacement for supervised fault labels.

5.7 Feedback and drift

After every inspection or repair, capture the actual fault, component, severity, work performed, replaced part and whether the alert was correct. Compare predictions with outcomes monthly. Monitor data drift (sensor distribution changes), concept drift (relationship between features and failures changes) and label/prior drift (failure frequency changes). Retrain only after reviewed data, time-based validation and approval; keep rollback to the previous model.

6. Stage 6 – Edge AI Deployment and Integrated Solution

6.1 Deployment option evaluation

| Option | Strength | Constraint / use |
| --- | --- | --- |
| TinyML | Very small models on microcontrollers; low power. | May be too restrictive for a multi-sensor gateway application with 4 GB RAM; suitable for sensor-side simple detection. |
| TensorFlow Lite | Optimized inference runtime and quantization support. | Good when model is compatible with its converter/runtime; validate operators and target platform. |
| PyTorch Mobile | PyTorch model workflow and deployment ecosystem. | Runtime footprint and current platform support must be checked for the chosen device/software version. |
| ONNX Runtime | Portable model interchange and execution across supported environments. | Requires operator compatibility and testing on each target execution provider. |

Selected route: train the model centrally and export to ONNX; deploy with ONNX Runtime on the industrial gateway. This supports portability across at least two practical execution environments (for example, x86-64 Linux and ARM64 Linux, subject to operator/runtime testing) and reduces dependence on one training framework.

6.2 Optimization sequence

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 7

1. Remove redundant inputs after feature-importance review. 2. Limit the model to a compact feature vector and fixed inference window. 3. Apply tree/model simplification where supported. 4. Test quantization or reduced-precision execution if supported by the selected model/runtime. 5. Benchmark accuracy, memory and latency on the actual gateway. 6. Sign and version the approved package. Expected effects are a smaller package and lower inference time; any accuracy loss must be measured and must not violate the macro-F1 and critical-recall acceptance gates.

6.3 Thirty-minute network-loss operation

Gateway detects loss of central connection
↓
Continue local sensor acquisition, quality checks and edge inference
↓
Generate local alarms and store event/feature packets in a bounded queue
↓
Retain short raw diagnostic ring buffer and prioritize critical events
↓
After reconnection: authenticate → synchronize queued records → resolve sequence gaps
↓
Check model/configuration version → download signed update if approved → staged validation → activate / rollback

The gateway must have sufficient local queue capacity for at least 30 minutes at the measured feature rate, with reserved space for critical alarms. The local alarm function must not depend on the cloud. Store model version, configuration version and timestamps with each prediction.

6.4 Requirement verification

| Requirement | Design response | Verification method |
| --- | --- | --- |
| Alert within 100 ms | Local gateway inference and alarm path; cloud not in critical path. | Measure sensor-observation-to-local-alarm latency under peak load; require p99 ≤ 100 ms. |
| 4 GB RAM / 20 GB storage | Compact feature model; bounded ring buffer and queue; local feature store. | Stress test with all assigned channels and 30-minute disconnection. |
| At least 90% transmission reduction | Illustrative feature traffic 10.24 kB/s vs 6.4 MB/s raw; 99.84% reduction. | Measure actual bytes on uplink including protocol overhead. |
| AI ≥ 90% validation metric | Macro-F1 ≥ 0.90 proposed; critical recall separately monitored. | Time- and machine-separated validation; report confusion matrix. |
| Scale 200 to 300 machines | Modular gateway groups, machine registry, horizontal platform scaling. | Load test 300-machine equivalent throughput and storage. |
| Two execution environments | ONNX model and ONNX Runtime portability plan. | Run identical test vectors on x86-64 and ARM64; compare outputs and latency. |

6.5 Integrated Industrial IoE architecture

Vibration / temperature / current / pressure / acoustic sensors + PLCs (200 → 300 machines)↓ Industrial Ethernet / industrial Wi-Fi / fieldbus 
Edge gateways (4 GB RAM, 20 GB storage each)
↓ timestamping + quality checks + filtering + windowing + feature extraction 
I local raw ring buffer I local feature store 
ONNX edge inference → local NORMAL / WARNING / CRITICAL alarms
↓ secure feature/event transmission and queued outage synchronization 
Plant IoE platform: time-series store + event store + selected raw object store + maintenance records↓
Grafana operator dashboard / Power BI management dashboard
↓
Maintenance work order → inspection / repair → verified outcome feedback
↓
Hadoop-compatible data lake + Spark batch/stream analytics → training/validation → model registry↓ signed, versioned ONNX model distribution to edge gateways

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 8

6.6 Phased implementation roadmap

| Phase | Work | Main risk | Mitigation / exit gate |
| --- | --- | --- | --- |
| 1. Pilot | Instrument representative CNC, motor and hydraulic machines; collect baseline and labels; validate latency, features and model. | Insufficient failure labels or noisy sensors. | Select varied machines; involve maintenance staff; verify sensor installation and data quality; proceed only after measured performance. |
| 2. Plant-wide | Deploy gateway groups, dashboards, work-order feedback and central storage across 200 machines. | Network congestion, inconsistent machine integration and false alarms. | Segment networks, standardize protocols/IDs, tune thresholds and roll out by production cell. |
| 3. Scale to 300 | Add gateway capacity, automated provisioning and ONNX deployment to x86-64/ARM64. | Capacity growth, model drift and platform incompatibility. | Load-test 300 machines, monitor storage/throughput, run cross-platform conformance tests and maintain rollback. |

6.7 Risk and mitigation register

| Risk | Practical mitigation |
| --- | --- |
| Cybersecurity | Network segmentation, device certificates, least-privilege access, encrypted transport, signed updates, patching and audit logs. |
| Model drift | Monitor feature distributions and outcome metrics; retrain only after reviewed labels and validation. |
| False alarms / missed faults | Use class-specific thresholds, human acknowledgement, critical-recall monitoring and pilot tuning. |
| Interoperability | Use documented protocols, common machine IDs, standard feature schemas and ONNX model exchange. |
| Storage growth | Retention tiers, compression, event-based raw capture, quotas and capacity alerts. |
| Network failure | Local inference, 30-minute queue, local alarms and reliable synchronization after recovery. |
| Vendor lock-in | Portable formats, open interfaces, exportable data and at least two tested runtime environments. |

7. Final Design Decision

APPROVAL RECOMMENDATION: YES – approve a controlled pilot, followed by gated deployment.

The design is technically defensible because the raw baseline is quantified at 6.4 MB/s and 368.64 GB per 16-hour day. The proposed illustrative feature packet produces 10.24 kB/s, a 99.84% reduction against continuous raw transmission. Local inference and local alarm generation address the 100 ms target and 30-minute connectivity-loss requirement. A hybrid storage policy separates selected raw diagnostic data from long-lived features and events. Spark is assigned to plant-level analytics and training preparation, while edge gateways handle immediate decisions. ONNX provides a practical route to cross-platform model execution.

Approval is conditional on evidence from the pilot: (1) p99 observation-to-alarm latency no greater than 100 ms; (2) at least 30 minutes of disconnected local operation; (3) measured uplink reduction of at least 90%; (4) macro-F1 at least 0.90 and acceptable critical-fault recall on a time-separated test set; (5) gateway memory/storage within 4 GB/20 GB; and (6) successful 300-machine load and cross-platform tests before full expansion. If any gate fails, revise sampling, gateway sizing, model complexity or thresholds before plant-wide deployment.

8. References and Design Basis

1. CO2 Application-Based Assignment: “Smart Manufacturing: Evaluate, Design and Deploy an AI-Driven Predictive Maintenance System”, ECA58 – IoE and AI Driven System, Unit II (the supplied assignment brief).

2. IEEE 2413 – Architectural Framework for the Internet of Things (reference framework identified in the supplied assessment rubric).

3. IEEE 2301 – Guide for Cloud Portability and Interoperability Profiles (reference identified in the supplied assessment rubric).

4. IEEE 7000 series – Ethical and trust considerations in system design (reference identified in the supplied assessment rubric).

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                             Page 9

5. Apache Hadoop, Apache Spark, Grafana, Microsoft Power BI, Tableau, ONNX and ONNX Runtime official technical documentation should be consulted during detailed implementation and procurement. Product versions and supported operators must be checked at deployment time.

Note: The weighted scores, packet format, thresholds and storage allowances in this report are explicit engineering proposals/assumptions for the assignment scenario. They are not measured plant results and must be validated during the pilot.

Jeeva R | 1912260266 | CO2 Application-Based Assignment                                                                            Page 10