ECA58 – IoE AND AI DRIVEN SYSTEM

CO1 – CONCEPT-BASED ASSIGNMENT

Design and Validation of a Connected Smart Hospital using Internet of Everything Technologies

Submitted by

Jeeva.R

Register Number: 1912260266

Course Outcome: Understand foundational IoE principles, framework components, enabling technologies and layered architecture.

Assessment: Design and Evaluation Report | Maximum Marks: 100

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 1

Executive Summary

This report proposes an integrated Internet of Everything (IoE) design for a 300-bed multi-speciality hospital. It connects people, processes, data and physical things rather than leaving cameras, building systems and barcode wristbands in separate departmental silos. The design combines NFC for bedside medication checks, RFID for contactless identity and selected equipment tracking, BLE for wearables and indoor location, low-power wide-area connectivity for remote oxygen sensors, and wired Ethernet/Wi-Fi for high-bandwidth clinical systems. Critical video analytics and alarms run on redundant on-premises edge servers so they continue during internet failure and can target sub-20 ms local processing. A fog layer coordinates wards and gateways; a cloud layer supports approved long-term analytics and backups. Open interfaces, segmented security, local fail-safe controls and staged deployment reduce risk and vendor lock-in.

Design Assumptions

• Baseline values below are planning assumptions for evaluation, not measured hospital results. A pilot must establish real baselines.

• Critical alerts are generated and processed locally; the 20 ms target refers to the local analytics-to-alert path, not the complete remote human response.

• Clinical decisions remain with qualified staff. AI and automation support decisions and trigger defined safety workflows.

• Patient-identifiable data remains on hospital premises by default. Any external transfer requires approval, legal review and access controls.

• Redundancy, monitoring, backup power and periodic failover tests are required to target 99.9% availability for critical alerts.

Stage 1 – Evaluate the Shift from IoT to IoE

1.1 Gap Analysis

| Pillar | Present state | Gap | Impact |
| --- | --- | --- | --- |
| People | Doctors, nurses and patients use separate identification methods. | No unified, role-aware identity and context across departments. | More manual checks and delays; increased risk of wrong-person or wrong-patient actions. |
| Process | Departments operate in silos; building dashboard is separate. | Medication, alarms, equipment search and facilities workflows are not integrated. | Slow escalation, duplicated work and unclear accountability. |
| Data | Camera, barcode and building data are isolated. | No common data model, timestamps, event rules or governed sharing. | Limited whole-hospital insight and weak analytics. |
| Things | Wi-Fi cameras, building systems and barcode wristbands exist. | Missing connected sensors, reliable gateways, location tags and automated feedback. | Blind spots in cold rooms, oxygen stores, occupancy and equipment tracking. |

Judgement: The current hospital is closer to an early IoT deployment than a complete IoE system. It has connected things, but the devices and departments work independently. IoE requires people, process, data and things to work together in shared, secure workflows. For example, a bedside scan should connect the patient identity, prescription, medicine record, nurse identity and administration process. The proposed design creates this integration while keeping safety-critical functions local.

1.2 Measurable operational impact

| Indicator | Assumed baseline | Target after deployment | Measurement method |
| --- | --- | --- | --- |
| Critical alert delivery | Typical local path 100 ms or more; not measured | Local analytics-to-alert under 20 ms; 99.9% availability | Timestamped event logs and monthly uptime |
| Mobile equipment search | Average 12 minutes per request | Average 4 minutes or less | Time-stamped equipment requests and location records |
| Medication verification | Manual checks and barcode workflow; error baseline to be measured | At least 50% fewer preventable verification mismatches reaching administration | Near-miss and medication incident reports; never bypass checks |
| Energy use | Current monthly electricity and water use to be established | 10–15% reduction in controllable HVAC/lighting energy in pilot areas | Meter readings normalised for occupancy and weather |
| Cold-room excursions | Detection and response time not consistently integrated | Alarm within 1 minute of threshold breach; backup cooling starts automatically | Sensor logs and alarm acknowledgement time |

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 2

Targets are proposed evaluation goals rather than guaranteed outcomes. Results should be reviewed after a 90-day pilot and adjusted using actual data.

1.3 Future trends and priority

Trend 1 – Edge AI and privacy-preserving analytics: More inference can run inside the hospital, reducing delay, bandwidth use and exposure of identifiable video. The main risks are model drift, false alarms and the need for local computing support.

This deserves first priority because the under-20 ms and offline requirements cannot safely depend on a remote cloud.

Trend 2 – 5G now and emerging 6G: 5G can support high-throughput mobile video, tele-ICU and low-latency services when coverage and network configuration are suitable. 6G capabilities such as sub-THz links and AI-native networking remain evolving and should not be treated as guaranteed near-term features. The roadmap should deploy 5G selectively while keeping interfaces and network equipment upgradeable for future standards.

Stage 2 – Design the IoE Framework (IEEE 2413)

2.1 Framework evaluation

IEEE 2413 provides a cross-domain architectural framework that helps organise devices, connectivity, data handling, applications and security across clinical, facilities, security and administrative domains. It encourages reuse and interoperability. However, a framework alone does not specify every clinical workflow, guarantee a vendor's implementation will interoperate, or automatically achieve latency, privacy and availability targets. The hospital must add detailed interface standards, clinical governance, threat modelling, safety validation and service-level tests.

2.2 Layered IoE architecture

Things / sensing layer: NFC readers, RFID readers and tags, BLE wearables and beacons, occupancy sensors, temperature/humidity probes, oxygen pressure/level sensors, smart meters, cameras, relays and cooling actuators.

Communication layer: Ethernet for fixed critical infrastructure; Wi-Fi for authorised mobile devices and high-data-rate coverage; BLE for wearable-to-phone links; NFC for tap verification; RFID for portal identification; Zigbee/IEEE 802.15.4 mesh for selected room controls; LoRaWAN or NB-IoT for remote low-rate sensors; secured IP backhaul.

Data and processing layer: local edge inference and event broker; ward/plant fog gateways; time-series database, device registry, rules engine, audit logging and approved cloud analytics.

Application and people layer: nurse dashboard, medication verification app, facilities dashboard, biomedical equipment locator, cold-room alarm console, tele-ICU interface and hospital operations reporting. People and processes span every layer through roles, approvals, escalation and feedback.

Architecture diagram

PEOPLE & PROCESS: clinicians | patients | facilities | administrators | safety workflows

APPLICATIONS: medication check | nurse alerts | equipment finder | HVAC | cold-chain | analytics | tele-ICU

DATA / EDGE / FOG: local AI + event rules | ward gateways | audit + time-series store | approved cloud

COMMUNICATION: Ethernet / Wi-Fi / BLE / NFC / RFID / 802.15.4 mesh / LoRaWAN or NB-IoT

THINGS: wearables | wristbands/tags | cameras | room sensors | meters | oxygen sensors | relays/cooling

Security, identity, monitoring, time synchronisation and resilience apply across all layers.

2.3 Pillar mapping

| IoE pillar | Hospital implementation | Example output |
| --- | --- | --- |
| People | Staff identity, patient identity, role-based access and nurse phone notifications. | Correct person receives the relevant alert. |
| Process | Medication rights checks, escalation rules, cold-room response and maintenance workflows. | A defined action follows each event. |
| Data | Patient context, prescriptions, vital summaries, location events and sensor history with access control. | Timely and auditable decisions. |
| Things | Sensors, tags, readers, wearables, cameras, actuators and gateways. | Physical conditions are measured and controlled. |

2.4 End-to-end flow A – bedside medication verification

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 3

1. Things: nurse taps phone on patient NFC wristband and scans/taps the medicine identifier. 2. Data: the app retrieves the patient identity, active order, medicine, dose, route and scheduled time from the hospital system. 3. Process: local rules compare the medication rights and check for allergy/duplicate/expired-order warnings where available. 4. Intelligence and decision: the rules engine flags a mismatch; the app blocks confirmation and asks for authorised review. 5. People: nurse checks the warning and contacts the prescriber/pharmacist if required. 6. Feedback: after a valid check and nurse confirmation, administration is recorded with a timestamp and audit trail. A scan alone must never be treated as proof that the medicine is clinically appropriate.

2.5 End-to-end flow B – vaccine cold-room monitoring

1. Things: calibrated temperature probe measures the cold-room temperature; a controller drives a local alarm and backup-cooling relay. 2. Data: readings are timestamped and sent to the local gateway. 3. Process: a rule compares the reading with the approved vaccine storage range and checks persistence to avoid nuisance alarms. 4. Intelligence and decision: the local controller decides whether to alarm and switch to backup cooling. 5. People: facilities and pharmacy staff receive the alert and acknowledge it. 6. Feedback: the system records temperature recovery, relay state and staff action. Network loss must not stop the local alarm or cooling control.

A weak pillar can defeat excellent technology. If staff are poorly trained or the process allows alerts to be ignored, a correct medication mismatch alert may still be overridden. Mitigation: role-based training, mandatory acknowledgement, escalation, audit review and a safe override process with reason capture.

Stage 3 – Enabling Technologies

3.1 Sensor / Actuator / Identification Selection

| Need | Candidates evaluated | Selected device | Justification | Risk |
| --- | --- | --- | --- | --- |
| Ward entrances / pharmacy / cabinets | Passive RFID, NFC, BLE | Passive RFID portals for rapid authorised tag reads; access control linked to staff identity | RFID offers hands-free portal reads; access decisions remain server/controller governed. | Missed or stray reads; use tuned antennas, door sensors and audit logs. |
| Bedside medicine check | Barcode, NFC, RFID | NFC wristband plus medicine barcode/NFC identifier on managed nurse phone | Short tap range reduces accidental reads and supports deliberate patient confirmation. | Phone loss or cloned tag; device management, authentication and server-side validation. |
| Equipment location | BLE beacons, RFID portals, UWB | BLE tags for room/zone location; RFID at selected exits | BLE gives practical battery-powered zone tracking; RFID portal events help detect movement. | BLE location is approximate; label confidence and use staff confirmation for critical equipment. |
| Ward occupancy and climate | PIR, mmWave, temperature/RH sensors | Privacy-conscious occupancy sensor, digital temperature probe and RH sensor | Supports occupancy-aware HVAC and comfort monitoring without relying on video. | Sensor placement/calibration can affect readings. |
| Lighting and HVAC | Manual switches, smart relays, building bus | Rated relay/contactor with local manual override and safe default | Enables control while preserving local operation and electrical isolation. | Wrong rating or unsafe switching; qualified installation and interlocks. |
| Vaccine/blood cold room | RTD/thermistor, digital probes; alarm/relay | Calibrated probe, local controller, buzzer and backup-cooling relay | Local closed-loop response works during network loss; accuracy and traceability are important. | Probe failure or stuck relay; dual-sensor plausibility checks and periodic tests. |
| Oxygen plant | Pressure transducer, level sensor | Industrial pressure transducer plus suitable cylinder/bank level sensing and local gateway | Measures pressure/remaining supply and supports threshold alarms. | Hazardous environment and calibration; use certified installation and independent safety controls. |

3.2 Suitable ranges and actions

| Measurement | Design range / target | Action |
| --- | --- | --- |
| Ward temperature | 0–50 °C sensor range; about ±0.5 °C accuracy or better | Notify facilities if outside configured comfort limits; HVAC response follows approved policy. |
| Relative humidity | 0–100% RH; about ±3% RH accuracy | Alert and adjust ventilation/HVAC if sustained outside configured range. |
| Bed occupancy | Occupied / unoccupied detection; validate false-positive rate in pilot | Adjust lighting/HVAC only within safety and patient-care rules. |
| Cold-room temperature | Choose probe covering the specified storage range; target ±0.5 °C or better | Local alarm and backup cooling on threshold breach; threshold set for each stored product. |

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 4

| Measurement | Design range / target | Action |
| --- | --- | --- |
| Oxygen pressure/level | Sensor range selected from actual manifold/cylinder specification; target about ±1% full-scale where justified | Low-pressure/low-level alarm and replenishment escalation. |
| Energy/water meters | Certified meter with pulse or digital output | Read every 15 minutes; flag abnormal consumption and leaks. |

Accuracy costs more and can increase calibration effort. For cold-chain and oxygen safety, favour reliable calibrated industrial-grade sensors over the cheapest option. For noncritical occupancy, a lower-cost sensor may be acceptable after a site trial. Exact oxygen ranges and vaccine thresholds must be set from equipment specifications and clinical/pharmacy policy, not guessed.

3.3 Cold-room sense–process–act node

CALIBRATED TEMPERATURE PROBE → LOCAL MCU/PLC (threshold + sensor-fault check) → RELAY/CONTACTOR → BACKUP COOLING

I LOCAL BUZZER / BEACON / EVENT LOG

LOCAL CONTROLLER → wired Ethernet or secured Wi-Fi → ward/facility gateway → dashboard

Power: mains with UPS; controller and alarm remain powered during brief outages. Fail-safe: on network loss, continue sensing, alarm locally and control backup cooling; buffer readings for later upload. On sensor fault, raise a fault alarm and invoke the documented safe procedure rather than assuming temperature is normal. Use manual override, relay-state feedback, calibration schedule and periodic failover tests.

Stage 4 – Computing Architecture: Cloud, Edge and Fog

4.1 Cloud provider evaluation

The hospital should run critical clinical control locally and select a cloud only for approved workloads. The following illustrative weighted matrix uses a 1–5 score (5 = strongest fit); weights prioritise security/residency and edge support. Scores are planning judgements that must be validated against contracts, region availability, current pricing and the hospital's compliance review.

| Criterion | Weight | AWS | Azure | Google Cloud |
| --- | --- | --- | --- | --- |
| Device management/connectivity | 15% | 5 | 5 | 4 |
| Analytics and AI services | 15% | 5 | 5 | 5 |
| Security/compliance controls | 20% | 5 | 5 | 5 |
| Data residency / deployment options | 20% | 4 | 5 | 4 |
| Edge support | 15% | 5 | 5 | 4 |
| Cost transparency / cost control | 5% | 3 | 3 | 4 |
| Portability / lock-in risk | 10% | 3 | 3 | 3 |
| Weighted score (out of 5) | 100% | 4.45 | 4.65 | 4.30 |

Azure is the illustrative first choice where hybrid identity and on-premises integration are strong priorities; AWS is a close alternative with broad IoT/edge capabilities; Google Cloud is strong for analytics and AI. This is not a universal ranking. The procurement decision should use a proof of concept, data-processing terms, local region availability, exit costs and interoperability tests. Keep application data in open formats, use standard APIs/MQTT where suitable, containerise portable workloads and avoid provider-specific dependencies in critical control paths.

4.2 Workload placement

| Function | Placement | Reason |
| --- | --- | --- |
| OT/ICU critical video alert under 20 ms | On-premises edge GPU/accelerator and local network | Avoids WAN latency; works offline; video remains local. |
| Ward HVAC control | Local controller with ward/plant fog gateway | Fast local response and autonomous fallback; cloud not required. |
| Hospital-wide monthly energy/equipment analytics | Fog aggregation plus approved cloud or on-prem analytics | Aggregates data efficiently; cloud provides scalable historical analysis if approved. |

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 5

| Function | Placement | Reason |
| --- | --- | --- |
| Long-term patient trends | Hospital-controlled clinical database; approved encrypted backup/analytics only | Access governed by clinical policy and residency approval; preserve audit and retention rules. |

4.3 Three-tier Edge–Fog–Cloud architecture

EDGE: cameras + local AI inference + alarm service + room controllers. Critical decisions stay local.

FOG: redundant ward/plant gateways + event broker + local data cache + identity/policy services.

CLOUD / CENTRAL DATA: approved de-identified analytics, backups, fleet management and long-term reporting.

If internet fails: edge analytics and local alarms continue; fog gateways buffer events and maintain ward control; cloud dashboards and remote support may become temporarily unavailable. After recovery, buffered data is synchronised with duplicate-event protection. Two local edge nodes, redundant switches/power, dual paths where feasible, UPS and monitored failover support the 99.9% critical-alert objective. The hospital must test the complete failure path; cloud availability alone cannot establish this target.

Stage 5 – Communication Network Design

5.1 Technology comparison

| Technology | Range / rate | Power and topology | Spectrum / coverage | Best fit and limitation |
| --- | --- | --- | --- | --- |
| Wi-Fi (802.11) | Room/building; high throughput | Moderate/high power; star via APs | Unlicensed; coverage affected by walls and congestion | Cameras, managed phones and high-rate clinical traffic; poor choice for tiny sensors needing years of battery. |
| BLE | Room/zone; low-to-moderate data rate | Very low power; star or beacon-based location | 2.4 GHz unlicensed; affected by interference | Wearables to nurse phones and equipment zone location; not a guarantee of precise location. |
| Zigbee | Building/room mesh; low data rate | Low power; mesh | 2.4 GHz or regional sub-GHz variants; mesh helps coverage | Lighting and HVAC control; requires reliable coordinator/gateway design. |
| 6LoWPAN / IEEE 802.15.4 | Short-range low-rate IP mesh | Low power; mesh or star | Usually unlicensed; constrained throughput | IPv6 reachability for constrained sensors and room controls; needs border router and careful security. |
| NB-IoT | Wide-area cellular; small messages | Low-to-moderate device power depending on coverage and reporting | Licensed cellular spectrum; carrier coverage required | Remote oxygen stores and difficult locations; recurring service and operator dependence. |
| LoRaWAN | Long range; small messages, low data rate | Very low power; star-of-stars through gateways | Unlicensed sub-GHz; site gateway can improve coverage | Remote oxygen readings; payload, duty-cycle and downlink constraints; private gateway may be needed. |

5.2 Weighted decision matrix

Weights: range/coverage 20%, power 20%, data rate 15%, topology 10%, security 15%, cost 10%, difficult-location resilience 10%. Scores 1–5 (5 best for the stated link). These are design estimates, not measured RF results.

| Link / candidate | Range | Power | Rate | Topology | Security | Cost | Difficult site | Weighted score |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wearable →nurse phone: BLE | 3 | 5 | 3 | 4 | 4 | 5 | 3 | 3.90 |
| Wearable →nurse phone: Wi-Fi | 3 | 2 | 5 | 3 | 4 | 3 | 3 | 3.25 |
| Room controls: Zigbee | 4 | 5 | 2 | 5 | 4 | 4 | 4 | 4.00 |
| Room controls: 802.15.4 + 6LoWPAN | 4 | 5 | 2 | 5 | 4 | 4 | 4 | 4.00 |

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 6

| Link / candidate | Range | Power | Rate | Topology | Security | Cost | Difficult site | Weighted score |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote oxygen: LoRaWAN | 5 | 5 | 1 | 4 | 4 | 4 | 5 | 4.05 |
| Remote oxygen: NB-IoT | 5 | 3 | 2 | 4 | 5 | 3 | 4 | 3.85 |
| Basement meters: Wi-Fi | 2 | 2 | 5 | 3 | 4 | 4 | 2 | 3.05 |
| Basement meters: wired Ethernet/RS-485 gateway | 4 | 4 | 3 | 4 | 5 | 4 | 5 | 4.15 |
| OT/ICU cameras: wired Ethernet | 5 | 2 | 5 | 5 | 5 | 3 | 5 | 4.30 |
| OT/ICU cameras: Wi-Fi | 3 | 2 | 5 | 3 | 4 | 3 | 3 | 3.25 |

Selected links: BLE for wearables; Zigbee or 802.15.4 mesh for room control after interoperability testing; LoRaWAN for low-rate remote oxygen sensors if a reliable private gateway can cover the 4 km site; wired Ethernet/RS-485 gateway for basement meters; wired Ethernet for OT/ICU cameras. Wi-Fi remains available for mobile devices and secondary coverage.

5.3 LoRaWAN versus NB-IoT for remote sensors and basement meters

For remote oxygen sensors, LoRaWAN can provide multi-year battery life and private coverage without a per-device cellular subscription, provided a site survey confirms link margin and local gateway/backhaul resilience. NB-IoT is preferable if cellular coverage is proven, managed operator service is desired, and the operator's service, security and cost terms are acceptable. For basement meters, first prefer wired Ethernet or RS-485 where installation is feasible because it avoids uncertain radio propagation. If wireless is required, NB-IoT may penetrate difficult locations better in some deployments, while LoRaWAN can be attractive with a well-placed gateway. The decision changes if the survey shows poor LoRa coverage, a carrier offers verified indoor coverage, local regulations constrain LoRaWAN use, or service cost/availability changes.

5.4 Five-year battery-life estimate for link (c)

Assume a 3.6 V, 19 Ah primary lithium battery; 24 reports per day; 20-byte payload; 2 seconds of radio activity per report at 40 mA; sleep current 0.015 mA; and 20% reserve for self-discharge, retransmissions and cold conditions. Daily radio charge = 24 ×2 s × 40 mA / 3600 = 0.533 mAh/day. Sleep charge = 0.015 mA × 24 h = 0.360 mAh/day. Total = 0.893 mAh/day. Five-year consumption = 0.893 × 365 × 5 = 1,630 mAh. With a 20% reserve, planned capacity is about 1,956 mAh, well below 19,000 mAh.

This simplified estimate suggests the five-year target is feasible, but it excludes sensor current, poor signal retries, battery derating and actual radio duty cycle. Include the pressure sensor's average current in the final calculation, test at the weakest-signal store and monitor battery state. If life is inadequate, reduce reporting frequency when safe, use adaptive reporting, improve gateway placement or increase battery capacity.

5.5 Role of 6LoWPAN and complete topology

IEEE 802.15.4 defines a low-rate wireless physical/MAC link; 6LoWPAN adapts IPv6 packets to constrained links. It is useful for room sensors that benefit from IP addressability and standard routing, but adds configuration and security-management complexity. Use it for selected constrained room-control nodes if the chosen products support interoperable IPv6 and secure commissioning; do not use it for high-resolution video.

Topology: NFC phone taps and RFID portals connect to secured access/clinical services; BLE wearables connect to managed nurse phones; room sensors/actuators form Zigbee/802.15.4 mesh to redundant ward gateways; oxygen sensors connect by LoRaWAN to a gateway at the remote site and secured backhaul to the hospital; basement meters use RS-485/Ethernet to plant gateways; cameras connect by wired Ethernet to redundant local edge switches and AI servers. Gateways connect over redundant segmented hospital Ethernet to fog services and approved cloud connections.

Single point of failure: one central network switch. Remove it with redundant core switches, dual uplinks, redundant power and tested automatic failover. Keep local safety control independent of the core network.

Stage 6 – 5G/6G Impact and Integrated Solution

6.1 Evaluation of 5G and 6G

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 7

5G eMBB can carry high-resolution tele-ICU video and rich clinical collaboration; URLLC aims at low latency and high reliability in suitable engineered deployments; mMTC supports large populations of low-rate connected sensors. Real performance depends on radio coverage, network configuration, device support, congestion, and end-to-end application design. A 5G link by itself does not guarantee a sub-20 ms alarm or safe remote procedure. Keep critical alarms local and use 5G as an additional communication path.

Within five years, private or managed 5G for tele-ICU, mobile clinical video and selected remote-assistance trials is plausible where coverage and governance are verified. 6G concepts such as sub-THz operation, AI-native networks and integrated sensing/communication are promising but speculative for a production hospital roadmap. Design for upgradeability rather than depending on these features.

6.2 Phased roadmap

| Phase | Deployment | Kept upgradeable | Key risk / control |
| --- | --- | --- | --- |
| Phase 1 – Today | Inventory devices; establish identity and data governance; install NFC medication verification pilot, calibrated cold-room sensors, local alarms, wired camera/edge AI pilot, segmented network and redundant core. | Open APIs, MQTT/standard data formats, containers, modular gateways and vendor-neutral device registry. | Integration and staff adoption; mitigate with pilot, training, clinical safety review and rollback plan. |
| Phase 2 – 5G | Add 5G coverage or private network for tele-ICU/mobile video; test QoS, coverage, failover and remote specialist workflows. | Modular radio/network interfaces and local clinical control independent of 5G. | Coverage or service-level gaps; use site survey, backup path and no dependence for local critical alarms. |
| Phase 3 – 6G-ready | Evaluate standards and trials when mature; update radio/core components only after validated clinical and security testing. | Replaceable radio modules, software-defined networking where appropriate, standards-based APIs. | Immature standards and cost; require evidence, interoperability testing and staged procurement. |

6.3 Integrated architecture

PATIENTS / STAFF / FACILITIES: role-based identity + approved clinical workflows

APPLICATIONS: medication safety | nurse alerts | equipment location | cold chain | utilities | tele-ICU

EDGE: on-prem video AI + local alarm controller + local cache (continues offline)

FOG: redundant ward/plant gateways + event broker + audit logs + policy enforcement

NETWORK: wired Ethernet for cameras/core | Wi-Fi managed devices | BLE wearables | NFC/RFID ID

LOW-POWER / ROOM LINKS: Zigbee/802.15.4 mesh | LoRaWAN or NB-IoT remote sensors | RS-485/Ethernet meters

CLOUD: approved encrypted backup + de-identified analytics + fleet reporting; external data only with approval

Feedback path: sensors → local rules/AI → authorised people → recorded action → actuator or workflow feedback. Critical functions do not depend on cloud connectivity.

6.4 Main risks and mitigations

| Risk | Mitigation |
| --- | --- |
| Cyberattack / ransomware | Network segmentation, device certificates, MFA, least privilege, patch management, immutable backups, monitoring and incident drills. |
| Patient privacy breach | Local video analytics, data minimisation, encryption in transit/at rest, role-based access, audit logs and approval for external transfer. |
| Interoperability / vendor lock-in | Open protocols and APIs, standard data formats, contractual data export, portability tests and avoid proprietary dependencies in safety paths. |
| Sensor failure / false alarms | Calibration, redundant critical probes, plausibility checks, health monitoring, local manual controls and scheduled proof tests. |
| Staff adoption / alert fatigue | Co-design with clinicians, training, sensible alarm thresholds, escalation policy and periodic review of false positives. |
| Power or network outage | UPS, redundant switches and servers, local controller autonomy, store-and-forward data and tested disaster recovery. |
| AI error / model drift | Clinical validation, human oversight, version control, performance monitoring, fallback rules and documented change approval. |

6.5 Validation plan

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 8

• Measure the full local critical-alert path under normal load and peak load; demonstrate under 20 ms for the defined analytics-to-alert boundary.

• Disconnect the internet and verify local alarms, HVAC rules, cold-room backup cooling and edge analytics continue; reconnect and confirm buffered events reconcile correctly.

• Run a 90-day pilot for equipment search time, medication verification, energy use, cold-room excursion response and user acceptance.

• Perform penetration testing, access-control review, restore tests, gateway battery tests and radio coverage surveys at every remote store and basement plant room.

• Report monthly availability and investigate every critical-alert failure; 99.9% availability permits about 8.76 hours of downtime per year, so the measurement window and service boundary must be defined.

Final Design Decision

Recommendation: YES — approve the design for phased implementation, subject to pilot validation and clinical safety approval. The design addresses all four IoE pillars by connecting identity, data, workflows and physical devices. Local edge processing supports the under-20 ms target and operation during internet failure; redundant infrastructure and fail-safe controllers support the 99.9% critical-alert goal. NFC, RFID, BLE, low-power wide-area links, wired Ethernet and selected mesh technologies are assigned according to the needs of each link rather than forcing one technology everywhere. The 5G roadmap is realistic, while 6G is kept as an upgrade path. Approval should be conditional on radio surveys, measured battery tests, cloud/residency review, staff training, security testing and evidence from the pilot. If any target fails, revise placement, network redundancy or workload location before hospital-wide rollout.

References

• IEEE Std 2413, Standard for an Architectural Framework for the Internet of Things (IoT).

• IEEE Std 1934, OpenFog Reference Architecture for Fog Computing.

• IEEE 802.11 family – Wireless LAN standards.

• IEEE 802.15.4 – Low-Rate Wireless Personal Area Networks.

• IETF RFC 6282 – Compression Format for IPv6 Datagrams over IEEE 802.15.4-Based Networks (6LoWPAN).

• Bluetooth Core Specification – Bluetooth Low Energy.

• LoRa Alliance, LoRaWAN specifications and regional parameters.

• 3GPP specifications for NB-IoT and 5G system capabilities.

• Hospital-specific sensor calibration, electrical safety, clinical medication, cold-chain and data-protection policies must be consulted during implementation.

Note: Scores, baselines and target improvements in this report are explicit engineering assumptions for the assignment. They should be validated against the hospital site, clinical policies, vendor specifications and measured pilot data.

Jeeva.R | 1912260266 | ECA58 – IoE and AI Driven System                                                             Page 9